# Example – subdividing land affected by an owners corporation

Plan under the Subdivision Act 1988

This is a guide to preparing a plan under section 32 of the Subdivision Act 1988, which requires the unanimous resolution of the owners corporation.

This example incorporates the Subdivision (Registrar’s Requirements) Regulations 2011 and includes items covered in other published examples.

## Key elements for preparing a plan under section 32 of the Subdivision Act 1988

Note: some of the points below are referenced on the pages that follow.

If a proposed plan affects land that is a member of an owners corporation, the plan must be prepared pursuant to section 32 of the Subdivision Act 1988.

A plan prepared under section 32 of the Subdivision Act 1988 may do one or more of the following:

subdivide and consolidate lots

create or decrease common property

include land outside the plan into the owners corporation

remove land from the owners corporation

create roads or reserves

create, vary or remove easements or restrictions

alter lot entitlement and liability (as a result of other actions under 32)

create, merge or dissolve owners corporations

remove restricted or accessory lot status (strata plans)

amend or cancel a scheme of development under the Cluster Titles Act.

Unanimous resolution of the owners corporation is required for the plan under section 32 to proceed (except when section 32AI applies). Evidence of the unanimous resolution of the owners corporation is provided to the council as part of the plan certification process.

The owners corporation is the applicant for plans under section 32. If there are multiple owners corporations affected, the unlimited owners corporation will usually be the applicant.

If a plan under section 32 reduces the area of common property, consents of everyone with an interest in the lots (i.e. mortgagees) are required.

Any new lots in a section 32 plan must have a new lot identifier, (one that hasn’t been used in the plan before). This also applies to minor boundary adjustments between parcels and car park swaps. The full extent of all lots (i.e. all parts) affected must be shown in the section 32 plan, even if those parts of the lot’s boundaries are not affected.

- Note: existing affected common property does not need to be re-numbered.

Each plan under section 32 must have a ‘purpose of plan’ notation. Each action or step that the plan is taking must be detailed and set out in order.

When a plan affects land outside the owners corporation, before it can be dealt with the plan purpose must begin with bringing the land into the owners corporation. If land is not going to be affected by the owners corporation as a result of the subdivision, then the final action will be to remove the land from the owners corporation.

If the section 32 plan deals with land that was previously common property (i.e. subdivides common property to create a lot), the land is still owned by the owners corporation (unless vested as a road or reserve). The affected land is unable to be removed from the owners corporation in the same plan, and must be transferred prior to being removed.

An owners corporation cannot be dissolved if it still owns land. Any common property disposed of through a section 32 plan must first be transferred out of the owners corporation’s name before the owners corporation can be dissolved in a separate ‘text only’ section 32 plan.

An owners corporation schedule, indicating the entitlement and liability of lots affected by the plan, must be included in the plan.

If a plan is incorporating part of common property into a lot, the section 32 plan is not to show the area of common property affected as hatched or otherwise denoted. The new plan should just show the new parcel in its entirety.

When dealing with a plan that includes boundaries defined by buildings, consideration must be given to the existing plan boundary definitions. The boundaries defined by the section 32 plan must be consistent with the original plan, or completely re-define the existing plan through compilation.

Compiled plan sheets should be prepared and supplied with the section 32 plan, if appropriate. Land Use Victoria may request the relevant compiled sheets per section 24(6) of the Subdivision Act 1988 prior to registration of the section 32 plan.

Plans under the Subdivision Act 1988 that affect land in a Strata Titles Act 1967 plan (RP or SP) or Cluster Titles Act 1974 plan (CS) should be drawn in accordance with current practices. This includes showing cross-sections of parcels situated above or below each other.

- Consideration should be given to the compiled plan in these cases, which may be complex and require redrawing of the entire Strata Titles Act 1967 plan. (Note: when Strata Titles Act 1967 plans include boundaries defined by buildings, but do not define a boundary location, the default location for that boundary is the median of the structure.)

Note: the following example is a three-lot owners corporation.The owner of lot 3 is re-aligning its boundary with neighbouring land outside the owners corporation. Lot 3 will be giving up some of their lot to their neighbour, resulting in a reduction of the area of land affected by the OC.

## Existing plan

13

## Section 32 plan

8

7

13

6

6

11

## Compiled Plan

14

| © The State of Victoria Department of Environment, Land, Water and Planning 2017 This work is licensed under a Creative Commons Attribution 4.0 International licence. You are free to re-use the work under that licence, on the condition that you credit the State of Victoria as author. The licence does not apply to any images, photographs or branding, including the Victorian Coat of Arms, the Victorian Government logo and the Department of Environment, Land, Water and Planning (DELWP) logo. To view a copy of this licence, Disclaimer This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication. | Accessibility If you would like to receive this publication in an alternative format, please telephone the DELWP Customer Service Centre on 136186, email customer.service@delwp.vic.gov.au, or via the National Relay Service on 133 677 www.relayservice.com.au. This document is also available on the internet at www.delwp.vic.gov.au. |
| --- | --- |